# Research Plan — Bimlendra — Weeks 1–13 (Oct 19, 2026 – Jan 15, 2027)

How to use this document. Part A is a Reference Sheet. It contains every model, equation, parameter, scenario definition, logging field, and measurement rule that the weekly tasks use. Each weekly task in Part B refers to Reference Sheet sections by number (for example “Sheet A3”). Keep Part A at hand every week. Each weekly task lists its purpose, background, starting point, implementation steps, parameters, experiment protocol, data to record, processing procedure, deliverables, and a self-check. Weekly deliverables are due on Friday; a one-page log (what was done, what failed, what is blocked) accompanies every delivery.

Software policy. All work is done in MATLAB/Simulink with the acados MATLAB interface. No Python is used. Vehicle-level validation uses dSPACE ASM in Simulink; real-time hardware tasks are marked provisional and will be confirmed later.

# Part A — Reference Sheet

## A1. Software environment and versions to record

- MATLAB and Simulink, release R2023b or newer (record the exact release).

- acados, MATLAB interface, pinned to one release tag (use the newest tagged release at the start of W1; record the tag and commit hash; do not update during the project). Install following docs.acados.org → “MATLAB/Octave interface”. acados bundles CasADi for MATLAB; record the CasADi version.

- C compiler used by MATLAB (mex -setup): MinGW-w64 or MSVC; record the compiler and version.

- dSPACE: ASM (Automotive Simulation Models), ModelDesk, VEOS, ControlDesk, ConfigurationDesk; record release versions.

- Every dataset and every report carries a fingerprint (Sheet A9).

## A2. Pendulum-on-cart model (used in W1)

This is the model of the acados example examples/acados_matlab_octave/getting_started/ (file pendulum_on_cart_model.m or equivalent in your acados release). It is reproduced here so that the task is self-contained.

States x=(p,θ,v,ω): cart position p [m], pendulum angle θ [rad] with θ=0 pointing up, cart velocity v [m/s], angular velocity ω [rad/s]. Control u=F [N], horizontal force on the cart.

Parameters: cart mass M=1 kg, pendulum mass m=0.1 kg, pendulum length l=0.8 m, g=9.81 m/s².

Dynamics:

p=v,  θ=ω,

v=- m l sinθ ω2+m g cosθ sinθ+FM+m-mcos2θ,

ω=- m l cosθ sinθ ω2+Fcosθ+(M+m) g sinθl (M+m-mcos2θ).

Optimal control problem: horizon Tf=1.0 s, N=20 shooting intervals, sampling time Ts=Tf/N=0.05 s. Stage cost 12∥y-yref∥W2 with y=(p,θ,v,ω,F), yref=0, W=diag(103,103,10-2,10-2,10-2). Terminal cost 12∥xN∥We2, We=diag(103,103,10-2,10-2). Constraint -80≤F≤80 N. Initial state x0=(0,π,0,0) (pendulum hanging down; the task is a swing-up).

acados settings: integrator ERK (explicit Runge–Kutta 4), sim_method_num_stages = 4, sim_method_num_steps = 3; nlp_solver_type = 'SQP'; qp_solver = 'PARTIAL_CONDENSING_HPIPM', qp_solver_cond_N = 5; hessian_approx = 'GAUSS_NEWTON'; nlp_solver_max_iter = 100 at code generation; tolerances nlp_solver_tol_stat = nlp_solver_tol_eq = nlp_solver_tol_ineq = nlp_solver_tol_comp = 1e-6.

## A3. Kinodynamic trajectory-following vehicle model

The model is the one published in Z. Wang, J. Zha, J. Wang, “Autonomous Vehicle Trajectory Following: A Flatness Model Predictive Control Approach With Hardware-in-the-Loop Verification,” IEEE Trans. Intelligent Transportation Systems, vol. 22, no. 9, pp. 5613–5623, 2021, Section II. You do not need the paper to implement it; everything required is below.

Reference path. A path is parameterized by its arc length s [m]. Along it are given: the path heading ψpr(s) [rad], its curvature κp(s)=dψpr/ds [1/m], the curvature rate dκp/ds, the reference speed vr(s) [m/s] and its derivative dvr/ds, and a steady-state sideslip correction

ψpvr(s)=- lr κp(s)+m lf vr(s)2 κp(s)2 Cy (lf+lr),

with derivatives dψpvr/ds and d2ψpvr/ds2 obtained by differentiating this expression (for constant κp and constant vr both are zero). Define the abbreviation θ=ψ+ψpvr(s).

States (5): x=(e,ψ,v,vy,γ). e [m] is the signed lateral distance from the vehicle center of gravity (CG) to the path (positive to the left); ψ [rad] is the heading error relative to the desired heading ψpr(s)+ψpvr(s); v [m/s] is the speed error along the path, v=s-vr(s); vy [m/s] is the lateral velocity at the CG in the body frame; γ [rad/s] is the yaw rate.

Inputs (3): u=(Fx,Fy,Mz): total longitudinal force [N], total lateral force [N], total yaw moment [N·m] at the CG, in the body frame.

Known signal. The longitudinal body-frame velocity vx [m/s] is treated as a known, measured quantity (it is not a state of the prediction model). In the prediction model it is held constant over the horizon at its value at the current sample.

Continuous-time dynamics (with θ=ψ+ψpvr(s) and s=v+vr(s)):

e=vxsinθ+vycosθ

ψ=γ-dψprds+dψpvrds (v+vr)

v=Fxmcosθ-Fymsinθ+dψprds (v+vr) e-dvrds (v+vr)

vy=Fym-vx γ

γ=MzIz

The station advances as s=v+vr(s); s is propagated alongside the states (it is needed to evaluate the path signals).

Plant model for closed-loop simulation. The plant used to close the loop in simulation is the same model (A3.1)–(A3.5) plus the longitudinal-velocity equation

vx=Fxm+vy γ,

integrated with fixed-step RK4 at 1 ms (50 substeps per 50 ms control period). This is a deliberate choice: in Weeks 2–8 the controller’s prediction model and the plant are intentionally identical except for (i) the plant integrates vx by (A3.6) while the controller freezes vx over the horizon, and (ii) the plant integrates at 1 ms with RK4 while the controller’s internal integrator uses one RK4 step of 50 ms per shooting interval. The purpose of the identical-model setting is to attribute every measured effect to the optimization and not to model mismatch. A high-fidelity validation with the dSPACE ASM vehicle model is a separate task (W9) with its own interface (Sheet A11).

Vehicle parameters (from the configuration used in Z. Wang & J. Wang, Control Engineering Practice 101:104482, 2020, Table 2): m=1723 kg, Iz=1960 kg·m², lf=1.232 m, lr=1.468 m, lateral tire stiffness Cy=62 700 N/rad (per tire), track width 1.539 m, wheel radius Rw=0.31 m, friction coefficient μ=0.9 unless a scenario family says otherwise.

## A4. Constraints and obstacle representation

All constraints are stated on physical quantities. Bounds that depend on vx use the frozen value for the current sample.

- Lateral-velocity and yaw-rate limits (stability envelope, as in the source paper): |vy|≤0.02 μ g vx and |γ|≤0.85 μ g/vx, with g=9.81 m/s². Soft (slack variables).

- Road bounds on e: emin≤e≤emax with values set by the scenario family (Sheet A6). Soft.

- Input bounds: Fx∈[-9000,4500] N, Fy∈[-9000,9000] N, Mz∈[-6000,6000] N·m. Hard.

- Obstacles. Each obstacle j is an ellipse in the path coordinates (s,e) centered at (so,j(t),eo,j) with semi-axes bj (along the path) and aj (lateral). The constraint at prediction stage i (time ti=tk+i Ts) is

ho,j=(ei-eo,jaj)2+(si-so,j(ti)bj)2-1 ≥ 0,

soft. The obstacle’s station over the horizon is predicted with its current speed and deceleration: so,j(ti)=so,j(tk)+vo,j(tk) (ti-tk)+12ao,j (ti-tk)2 (clipped at zero speed). The ego station over the horizon is predicted as si=sk+l<i​Ts (vl+vr(sl)) using the previous solution’s v (or vr alone at the first sample). Both enter the OCP as stage parameters. Semi-axes: aj=1.3 m, bj=4.5 m (half vehicle width 0.9 m + 0.4 m margin; half length 2.5 m + 2.0 m margin). - Slack penalties on all soft constraints: linear weight z=103 and quadratic weight Z=103 per unit of violation (acados fields zl, zu, Zl, Zu).

## A5. Nonlinear MPC formulation (Member 1, “NMPC”)

Sampling time Ts=0.05 s, horizon N=40 (2.0 s preview). Shooting discretization: explicit RK4, one integration step of length Ts per shooting interval (sim_method_num_stages = 4, sim_method_num_steps = 1).

Decision variables: xk+i|k, i=0..N, and uk+i|k, i=0..N-1. Initial condition xk|k=x(k) (measured). Stage parameters at stage i: pi=(vx, si, ψpvr(si), dψpr/ds, dψpvr/ds, d2ψpr/ds2, d2ψpvr/ds2, vr(si), dvr/ds, obstacle data (so,j(ti),eo,j,aj,bj)j=1,2, emin,emax).

Cost (nonlinear least squares): with yi=(e,ψ,v,vy,γ,Fx,Fy,Mz)k+i|k and reference yiref=(0,0,0,0,0,Fxr,Fyr,Mzr),

J=i=0N-112 ∥yi-yiref∥W2+12 ∥xk+N|k∥We2+slack penalties,

W=diag(20, 50, 2, 0.1, 0.1, 10-6, 10-6, 10-7), We=10⋅diag(20,50,2,0.1,0.1). The feedforward references are the inputs that keep the vehicle exactly on the path at the reference speed:

Fxr=mcosψpvr dvrds vr+sinψpvr κp vr2, Fyr=mcosψpvr κp vr2-sinψpvr dvrds vr,

Mzr=Izd2ψprds2+d2ψpvrds2vr2+dψprds+dψpvrdsdvrds vr.

No terminal constraint is used (the source paper’s practice); stability is not certified by construction.

Solver settings: nlp_solver_type = 'SQP', qp_solver = 'PARTIAL_CONDENSING_HPIPM', qp_solver_cond_N = 10, hessian_approx = 'GAUSS_NEWTON', regularize_method = 'CONVEXIFY' (if available in your release; otherwise levenberg_marquardt = 1e-4), globalization = 'FIXED_STEP' with step length 1, nlp_solver_max_iter = 200 at code generation, tolerances 10-6 (all four), qp_solver_iter_max = 100. Cost scaling: the stage costs must not be multiplied by the time step. If your release exposes solver_options.cost_scaling, set it to a vector of ones of length N+1. In every case run the check of W3 step 5 to confirm.

Warm start. The solver’s internal iterate carried from one sample to the next is denoted z(k); it is the stacked primal trajectory z=[ xk|k;…;xk+N|k; uk|k;…;uk+N-1|k ] (plus multipliers inside the solver). The shifted warm start for sample k+1 is formed from the solution at k by dropping the first stage, moving every stage one index earlier, and duplicating the last stage: xk+1+i|k+1init=xk+1+i|k for i=0..N-1, xk+1+N|k+1init=xk+N|k, and likewise for u. Set it with set('init_x', ...), set('init_u', ...) (and init_pi if your release supports it).

Command the iteration cap. Before each solve, set the maximum number of SQP iterations: ocp_solver.set('nlp_solver_max_iter', cap) (if your release rejects this field name, try ocp_solver.options_set('nlp_solver_max_iter', cap); the value must not exceed the code-generation value of 200). The cap is denoted l; the number of iterations actually performed is read from get('sqp_iter') and denoted l.

## A6. Scenario families and randomization

One scenario instance is a complete specification of: the road (curvature κc, lane width, road bounds), the friction coefficient μ, the reference speed vr, the ego initial state, and the two other vehicles’ initial positions and motion. One run is an 8.0 s closed-loop simulation of that instance at Ts=0.05 s (160 control samples), with a fixed-step RK4 plant at 1 ms.

Common to all families: lane width 3.75 m; the reference path is the center line of the ego’s lane; the ego starts at s=0 with e0∼U[-0.3,0.3] m, ψ0∼U[-2°,2°], vx,0∼U[20,30] m/s, vy,0=0, γ0=0, v0=0 with vr(s)≡vx,0 (constant reference speed; the controller must slow down or change lane by itself). Lead vehicle (obstacle 1) in the ego lane: eo,1=0, initial gap d0∼U[25,45] m, initial speed equal to vx,0, decelerating at alead∼U[-6,-3] m/s² starting at t=1.0 s until it stops. Target vehicle (obstacle 2) in the left lane: eo,2=+3.75 m, constant speed vt∼U[vx,0-10, vx,0-3] m/s, initial gap dt (family-specific).

| Family | Road | μ​ | Road bounds [emin,emax] | Target vehicle gap dt | Instances |
| --- | --- | --- | --- | --- | --- |
| F1 straight, lane change available | κc=0​ | 0.9 | [-1.75,5.25] m | U[15,40] m | 300 |
| F2 curved, lane change available | κc drawn uniformly from {±1/600,±1/400,±1/300} 1/m | 0.9 | [-1.75,5.25] m | U[15,40] m | 300 |
| F3 straight, low friction | κc=0​ | 0.6 | [-1.75,5.25] m | U[15,40] m | 300 |
| F4 straight, no free lane | κc=0​ | 0.9 | [-1.75,1.75] m | U[-5,5] m (target beside the ego; the left lane is blocked; the ego must brake) | 300 |

Random seeds: instance i of family F uses rng(1000*F + i, 'twister') before drawing its parameters, in the fixed order: e0,ψ0,vx,0,d0,alead,vt,dt,κc. Record every drawn value.

Run termination: after 8.0 s, or earlier if the ego’s CG enters an obstacle ellipse with margins removed (semi-axes 0.9 m × 2.5 m), which is recorded as a collision, or if |e|>6 m (run-off). Terminated runs are kept in the dataset and flagged.

## A7. Per-solve logging schema

Every controller execution (every solve at every sample of every run) produces one record with the fields below. Store one MAT file per run containing a struct with arrays indexed by sample; also export a CSV of the scalar fields. Units in SI; angles in rad.

| Field | Content |
| --- | --- |
| family, instance, k, t | family id, instance id, sample index, time |
| x (5), vx, s | measured state, longitudinal speed, station |
| x_pred_prev (5) | the state predicted for this sample by the previous solve, xk¦k-1 (second stage of the previous solution) |
| dx (5), dx_norm | δxk=x(k)-xk¦k-1 and its scaled norm (Sheet A8) |
| u_applied (3) | input applied to the plant |
| member | which controller formulation produced it (1 = NMPC; later 2, 3, 4) |
| cap, sqp_iter, qp_iter (vector per SQP iteration), status | iteration cap l, iterations performed l, QP iterations, exit status |
| time_tot, time_lin, time_qp | solver-reported times [s] (as available in your release) |
| cost | objective value of the returned solution |
| res_stat, res_eq, res_ineq, res_comp | final KKT residuals |
| res_hist (ℓ × 4) | residuals after each SQP iteration (from the solver’s statistics table) |
| slack_max | maximum slack value over the horizon |
| active (vector) | for each soft/hard constraint: 1 if active at the solution (constraint value within 10-4 of its bound, in scaled units) |
| znorm_prev | ∥z(k)-z(k-1)∥ scaled (Sheet A8) |
| znorm_opt | ∥z(k)-z(k)∥ scaled, where z(k) is the converged solution of the same problem from the same warm start (available only when the offline re-solve of Sheet A8 has been run) |
| lam0 (5) | multiplier of the initial-state constraint (Sheet A8) |
| K0 (3 × 5) | sensitivity of the first input to the initial state (Sheet A8), computed on a subsample |
| e_stale | ∥K0 δxk∥ scaled (Sheet A8) |
| kappa_input, kappa_delay | sensitivity scalars (Sheet A8) |
| obstacles | obstacle positions and speeds at tk |
| fingerprint | Sheet A9 (stored once per file) |

## A8. Definitions of derived quantities

Scaling. Inputs are scaled by Uscale=(9000,9000,6000) (the half-ranges of the hard input bounds). States are scaled by Xscale=(1,0.1,5,1,0.5) (m, rad, m/s, m/s, rad/s). A “scaled norm” of a vector means the Euclidean norm after dividing each component by its scale.

Quality loss of a truncated solve, ε. For a given problem instance (fixed x(k), parameters, and warm start z(k)): let u* be the first input of the solution obtained with cap 200 and tolerances 10-8 (the converged solution) from that warm start, and u(l) the first input returned with cap l from the same warm start. Then ε(l)=∥u(l)-u*∥ (scaled norm). Two variants are recorded: εinst, where u* is converged from the same warm start; and εref, where u* is converged from a canonical cold start (all states equal to x(k) for all stages, all inputs equal to the feedforward references). Basin mismatch: if the two converged first inputs differ by more than 0.05 (scaled), flag the instance; keep it in the dataset.

Target quality level εtarget=0.01 (1 % of the input half-range).

Iterations needed lneeded: for an instance and a warm start, the smallest cap l∈{1,2,…,40} for which εinst(l)≤εtarget; equal to 41 if none qualifies. Unit: SQP iterations. It is obtained experimentally by sweeping the cap on the frozen instance (W4 protocol).

Initial-state multiplier λ0∈R5. The initial condition xk|k=x(k) is imposed in acados as equal lower and upper bounds on the stage-0 state. The multipliers of these bounds are available from get('lam', 0). With the acados ordering of stage-0 multipliers [lbu lbx lg lh ... ubu ubx ug uh ...], λ0=λubx-λlbx (the 5 entries associated with the state bounds). Interpretation: λ0 is the gradient of the optimal cost with respect to the current state. Verification (W3): compare with the central finite difference (J*(x(k)+hej)-J*(x(k)-hej))/(2h), h=10-3 Xscale,j, where J* is the converged optimal cost returned by get_cost().

First-input sensitivity K0∈R3×5: K0=∂uk|k*/∂x(k), computed by central finite differences: for each state component j, re-solve to convergence from the same warm start with x(k)±hej, h=10-3 Xscale,j, and take the difference quotient of the first input. Cost: 10 converged solves per instance; compute on every 8th sample.

Prediction error δxk=x(k)-xk|k-1 and stale-plan error estale(k)=∥K0 δxk∥ (scaled input norm).

Sensitivity scalars. With f the right-hand side of (A3.1)–(A3.5) and ∂f/∂u its Jacobian with respect to u (evaluated at x(k), u(k), from the CasADi model): κinput=∥λ0⊤ ∂f/∂u∥ (3-vector norm), κdelay=|λ0⊤f(x(k),u(k))|. Both are first-order sensitivities of the optimal cost: to an input error, and to the state drifting for one sample while the input is frozen.

Closed-loop cost over a window of Neval samples starting at k: Jcl=i=0Neval-112∥y(k+i)-yref(k+i)∥W2 evaluated on the plant trajectory (same W as Sheet A5, no terminal term), plus the slack-penalty value of any violated soft constraint on the plant trajectory.

Normalized solve time. On a given machine and session, compute the reference time per iteration cref = median over the first 500 instances of family F1 of (time_tot / sqp_iter) with cap 10. The normalized time of any solve is τ=time_tot/(cref l), dimensionless; values above 1 indicate iterations that were more expensive than the reference (for example more QP iterations). τ is reported alongside raw time; raw time is hardware-specific.

## A9. Fingerprint (record with every dataset and report)

CPU model and nominal clock; number of cores; RAM; operating system and version; Windows power plan (set to “High performance” for all timing work); whether CPU affinity was set (set MATLAB’s process affinity to one physical core for all timing work; record which); MATLAB release; acados tag/commit; CasADi version; compiler and version; compiler optimization flags (record the flags MATLAB’s mex uses, default -O2 for MinGW); BLASFEO target (GENERIC, X64_INTEL_HASWELL, …); HPIPM version; Simulink solver settings; date and time; machine name.

## A10. Timing measurement rules

- Power plan “High performance”; close other applications; MATLAB process affinity to one core; no other heavy process running unless the experiment is about contention.

- Warm-up: discard the first 200 solves of every session.

- Report medians and the 5th/95th percentiles; keep all raw values; never delete outliers (flag them).

- Timer: use the solver’s own time_tot for solver time and MATLAB tic/toc around the full controller step (including parameter updates and the shifted warm start) for the controller-step time; report both.

## A11. High-fidelity validation interface (ASM) — used in W9

The validation plant is the dSPACE ASM vehicle dynamics model running in Simulink (offline simulation), parameterized in ModelDesk to match Sheet A3 as closely as possible (m, Iz, lf, lr, front and rear tire cornering stiffness, track width, wheel radius). The controller outputs (Fx,Fy,Mz) must be converted to ASM actuator inputs by an allocation layer:

- Total drive/brake torque Ttot=Fx Rw, split equally between the two driven axles’ wheels (ASM: per-wheel torque inputs).

- Yaw moment by torque vectoring: left/right torque difference ΔT=Mz Rw / btrack added to the right wheels and subtracted from the left wheels.

- Front steering angle from the lateral force demand using the linear tire relation: δf=Fy2 Cy+vy+lf γvx (the second term is the front-axle slip angle at zero steering; it makes δf produce the requested front lateral force in the linear range), saturated to ±30°.

ASM’s own states (vx,vy,γ, position, heading) are converted to the controller’s states (e,ψ,v) by projecting the CG position onto the reference path. The sources of mismatch between controller model and ASM — tire saturation, load transfer, actuator dynamics, powertrain limits, the allocation approximation — are deliberately present in this validation.

# Part B — Weekly tasks

## W1— acados on a pendulum: create, configure, solve, interrogate

Purpose. Establish that you can create, configure, solve, and fully interrogate an acados optimal-control problem from MATLAB, including every API feature the vehicle work will need: commanding the iteration cap, reading iteration counts and residuals, reading multipliers, warm-starting, and building a Simulink S-function. The vehicle problem starts only after this works.

Background. acados solves nonlinear model predictive control (MPC) problems by sequential quadratic programming (SQP): each iteration linearizes the dynamics and constraints along the current guess of the trajectory, solves a quadratic program (QP), and updates the guess. The quantities you will read — the number of SQP iterations, the QP iterations inside each, the Karush–Kuhn–Tucker (KKT) optimality residuals (stationarity, equality, inequality, complementarity), the cost, and the Lagrange multipliers — are the solver’s own record of how hard each problem was and how close to optimal the returned solution is.

Starting point. Install acados (Sheet A1) and run examples/acados_matlab_octave/getting_started/minimal_example_ocp.m (name may differ slightly by release) unchanged; it must run before you continue. Then build your own script from Sheet A2.

Implementation steps. 1. Implement the model of Sheet A2 as an AcadosModel (CasADi symbolics for x, u, and x). 2. Set up the OCP with the cost, bounds, horizon, discretization, and solver settings of Sheet A2. Generate and compile the solver (AcadosOcpSolver). 3. Single solve from x0=(0,π,0,0) with an all-zero initial guess. Record: status, sqp_iter, the vector of qp_iter, time_tot, cost, final residuals, and the full statistics table. 4. Iteration-cap sweep on this one problem: for cap ∈{1,2,3,5,8,12,20,50,100}, set the cap (Sheet A5 “Command the iteration cap”), solve from the same all-zero initial guess, and record the first input F0(cap), the cost, the status, and the residuals. Confirm that the status reports “maximum iterations reached” exactly when sqp_iter equals the cap. Compute |F0(cap)-F0(100)| and plot it against the cap on a log scale. 5. Warm start: solve to convergence; form the shifted warm start (Sheet A5); set x0 to the second state of the converged trajectory; solve again and record sqp_iter. Compare with a cold (all-zero) start from the same x0. 6. Multipliers: read get('pi', i) for i=0..N-1 and get('lam', 0). Compute λ0 as in Sheet A8 and verify it against the finite-difference gradient of the optimal cost (Sheet A8 verification formula; report the relative error for each of the four state components). 7. Closed loop: simulate 4 s of swing-up with the plant integrated by RK4 at 1 ms and the controller at 50 ms, shifted warm start every sample. Plot p,θ,v,ω,F against time and sqp_iter against time. 8. Edge cases, each with a one-line result: (a) x0=(0,0,0,0) (already upright): expect convergence in very few iterations and F≈0; (b) F bound reduced to ±5 N: expect many active bounds and a different swing-up; (c) horizon N=5: expect failure to swing up within the window; (d) cap = 1 in closed loop (this is the real-time iteration in effect): report whether the swing-up still succeeds; (e) call solve() twice without changing anything: sqp_iter must be 0 or 1 on the second call. 9. Simulink: generate the S-function following examples/acados_matlab_octave/getting_started/simulink_example.m; build a Simulink model with the S-function and a plant block (RK4 at 1 ms, implemented as a MATLAB Function block or a Simulink integrator chain); reproduce the closed-loop result of step 7 within plotting accuracy; record the S-function’s sqp_iter, status, and CPU_time output ports. 10. Record the fingerprint (Sheet A9).

Data to record. For steps 3–8: a MAT file w1_results.mat with the recorded fields, plus a CSV table of the cap sweep (cap, sqp_iter, status, cost, residuals, F0, |F0-F0(100)|, time_tot).

Processing. Script w1_plots.m producing: cap-sweep plot (step 4), cold-vs-warm bar (step 5), multiplier verification table (step 6), closed-loop figure (step 7), Simulink-vs-script overlay (step 9).

Deliverables. w1_pendulum_model.m, w1_ocp_setup.m, w1_experiments.m, w1_plots.m, w1_results.mat, w1_cap_sweep.csv, Simulink model w1_pendulum_sfun.slx, figures as PNG and FIG, and a 2-page w1_report.md with the numbers from steps 3–9 and the fingerprint. A reader with your files and the Sheet must be able to regenerate every figure.

Self-check (evidence to report). The example from acados ran unchanged. Status equals “max iterations” iff sqp_iter = cap. |F0(cap)-F0(100)| decreases with the cap. Relative error of λ0 versus finite differences below 10-3 for all four components. Simulink and script closed loops agree.

## W2 — Vehicle model, scenario generator, and the first closed loop

Purpose. Build the simulation architecture used for all later data: a Simulink closed loop in which the NMPC of Sheet A5 (acados S-function) controls the kinodynamic vehicle model of Sheet A3, and a scenario generator that produces the four families of Sheet A6 reproducibly. Verify the architecture on one nominal instance per family.

Background. The architecture is: scenario and path signals → NMPC (acados S-function; prediction model = Sheet A3 with frozen vx) → inputs (Fx,Fy,Mz) → plant (same equations plus (A3.6), RK4 at 1 ms) → measured states fed back. The controller’s prediction model and the plant are intentionally identical in this phase (Sheet A3 explains exactly what differs). The high-fidelity ASM plant is used in W9, not here.

Starting point. Your W1 Simulink model and scripts; Sheets A3–A6.

Implementation steps. 1. Implement the vehicle model (A3.1)–(A3.5) as an AcadosModel with the stage parameters of Sheet A5 and the constraints of Sheet A4 (soft constraints with slacks; obstacle constraints as nonlinear path constraints h≥0 for two obstacles). Implement the cost of Sheet A5 including the feedforward references. Generate the solver with the settings of Sheet A5. 2. Implement the plant as a Simulink block (MATLAB Function block with RK4 at 1 ms, states (e,ψ,v,vy,γ,vx,s)). 3. Implement the path-signal and obstacle-prediction block: given sk, vx, the previous solution’s v trajectory, and the obstacle states, compute the stage parameter vectors p0…pN of Sheet A5. 4. Implement the scenario generator gen_instance(F, i) exactly as Sheet A6 specifies (seeds, draw order, families), returning a struct with all parameters; and run_instance(inst, member, options) that runs the 8.0 s closed loop and returns the log of Sheet A7 (for this week the fields that do not need the offline re-solve: leave znorm_opt, K0, e_stale, kappa_* empty). 5. Run instance i=1 of each family with the NMPC at cap 200 (converged). Plot: trajectory in (s,e) with the obstacle ellipses drawn at three times (0, 2, 4 s) and the road bounds; states and inputs against time; sqp_iter, time_tot, and the number of active constraints against time; slack maximum against time. 6. Reproducibility test: run instance (F1, 1) twice; the logs must be bit-identical (or identical to 10-12 if the solver is multithreaded — record which).

Parameters. All from Sheets A3–A6. Simulation 8.0 s, Ts=0.05 s, plant step 1 ms.

Data to record. The four nominal-run logs (Sheet A7) as MAT files w2_run_F{F}_0001.mat, the four instance structs, the figures.

Processing. w2_plots.m generating the figures of step 5 for any run file.

Deliverables. vehicle_model.m (acados model), ocp_nmpc_setup.m, plant_rk4.m (or the Simulink block), path_signals.m, gen_instance.m, run_instance.m, Simulink model vehicle_cl.slx, w2_plots.m, the four run files, figures, w2_report.md (one page per family: what the vehicle did; which constraints became active; where the solver needed the most iterations), and the fingerprint.

Self-check. Each nominal run completes 8.0 s without collision for F1–F3; F4 ends with the ego braking behind the lead vehicle without collision (if a collision occurs, report the time and the active constraints at that time — do not tune the scenario). The reproducibility test passes. sqp_iter is not constant over a run (report its minimum, median, and maximum per family).